This unit is designed to meet the latest ATEX requirements for hazardous area equipment and is approved for installation and uses in Zone 1 IIB T4 areas.

Although primarily designed for cranes, making it ideal for offshore use, the unit can be utilised in any suitable approved area.

The fully configured system consists of two enclosures, control unit and main unit.

The main unit is an ExD enclosure manufactured in aluminium alloy.

This houses the two radios (VHF, UHF) a universal input mains power supply, interface control electronics and audio output stages, to drive external speakers, and IS barriers.

All cable entries are via 20mm cable glands.

The optional PA is housed in the main unit and comprises of a 25-watt loud hail amplifier, this allows the operator to make announcements to deck personnel.

The control unit consists of a polycarbonate enclosure fitted with a LCD display to indicate channel number and mode, LED indicators to indicate status and push button switches to adjust volume levels and change channels on each radio module. Large push buttons are utilised which can be easily operated even with a gloved hand.

Cable entries to the main unit and foot/hand operated PTT are via 20mm cable glands. Connections to a system approved headset/boom microphone or gooseneck microphone is via a 5-pin connector.

SPECIFICATIONS

SIRA 03ATEX2404X

0518

# Radio Section     Freq VHF (136-174MHz, UHF 403-512MHz)

| Number of Channels | 16 per radio |
| --- | --- |
| Transmitter output | 1 Watt |
| Channel spacing | +/- 12,5 or 25kHz (specify when order) |
| Receiver Sensitivity (VHF) | 0.25uv 12dB SINAD |
| Receiver Sensitivity (UHF) | 0.3uv 12dB SINAD |
| CTCSS coding on UHF | optional |
| Audio Output | 5 Watts into External Speaker |

PA Section (optional)

| Loud Hail Output | 25 watts |
| --- | --- |
| Output | 100v Line |

# Main Enclosure Details

| Input Supply | 86-260v AC 65 Watts |
| --- | --- |
| Size (Excluding Glands) | 435x330x220 |
| Weight | 22kg (24kg PA Option) |
| Material | Aluminium Alloy |

# Control Enclosure

| Size (Excluding Glands) | 190x190x130 |
| --- | --- |
| Weight | 2kg |